# Residential Power Disaggregation - User Guide

Sponsored by ABB

Andrew Bailey, Ralph Cullom, Manny Harris, Labib Kasim

Main Components

Hardware Overview

-RaspberryPi

-Windows PC

-ESP32

-CT Boards

Home Assistant

-ESPHome

-InfluxDB

-Grafana

Machine Learning Application

-sched.exe

Table of Contents

Residential Power Disaggregation - User Guide	1

Initial Setup	3

Hardware	3

Raspberry PI	3

ESP32 and CT Boards	5

Software	5

Home Assistant	5

General Use	6

Home Assistant	6

Windows Application	6

# Initial Setup

## Hardware

#### Raspberry PI

- The Raspberry Pi 3 must be powered by a micro USB connection for a power supply

- A network connection is required and can be supplied through an ethernet cable

  - For a normal home network, connect an ethernet cable between a router and the raspberry pi. The ESP32 must be on the same network through WiFi for data collection

  - For use on eduroam without the ESP32, a connection can be provided by sharing a laptop’s network through ethernet

    - Find and open network connections

    - Right click on the wifi connection when you are on eduroam and choose properties

    - Allow other connections and choose ethernet as the method then apply these settings

  - ESPHome cannot be used on eduroam with homeassistant. To use them together on campus, an external network is needed. By using a WiFi hotspot on a phone a router can be implemented as a wireless bridge

    - Configure the router as a wireless bridge and connected to the hotspot network created

      - The router GUI can be opened by plugging an ethernet cable into a LAN port and connecting it to a laptop

      - Then visit 192.168.0.1 and log in with username and password, both “admin”

    - The router will create a new network called “Encore2” that the ESPHome can be connected to over WiFi as well as a computer through ethernet connected to a LAN port

#### ESP32 and CT Boards

- The ESP32 can be connected to the CT boards using the header sockets on the top.

- The CT boards will receive power from the power supply which will power the ESP23 as well

- The CTs themselves are plugged into the 3.5mm jacks on the sides of the CT board.

  - Calibration for the CTs can be found as comments in the code

For more information on the CT board layout see this github

## Software

#### Home Assistant

- The home assistant operating system is already set up on the SD cards in both the Raspberry Pi 3 and 4

  - For information on how to reinstall the OS, install add-ons, or configure ESPhome in home assistant see 1.2 software/home assistant

# General Use

#### Home Assistant

- After the Raspberry Pi is connected to the internet and powered on (see initial setup) you can access the GUI from a computer on the same network

- Access the home assistant interface by visiting the following link in a web browser http://homeassistant.local:8123

  - If necessary, you can instead use http://(IP Address):8123, where you replace it with your address i.e. 192.168.x.x

- Log in using these credentials

  - Username: seniordesign50

  - Password: seniordesign50

- On the left-hand side of the home assistant GUI, click on the Grafana tab

- Once in the Grafana GUI, click on the energy dashboard

- The dashboard will display the total power usage, predicted values, and cost calculations

- The time frame for viewing can be changed in the top right corner

#### Windows Application

- After home assistant is up and running, you can run the Windows application sched.exe from the same computer on the network

- The application will open up a command prompt window and ask you for credentials

  - The influx DB token is like a special password that allows you to access a certain bucket. For the raspberry pi 3, the influx DB token is: NZ3a8rOhDgkAb3TUj68oPDJaXlg8IL_EfhVhk5sWaTeF7oBcLbjLO5AscRo2y0mjw149K6uu9xNLieFSxE-lXA==

  - The URL is where you can access the influxDB database. With Home Assistant it is on port 8086. The url is: http://homeassistant.local:8086

- The program will make initial predictions for the past 20 minutes and then continue to make new predictions every following 20 minute